Journal of Advances in Modeling Earth Systems

Supporting Information for

Modeling urban traffic heat flux in the Community Earth System Model: Formulation and validation for two test sites

Yuan Sun1,*, Keith W. Oleson2, Zhonghua Zheng1,*

1Department of Earth and Environmental Sciences, The University of Manchester, Manchester M13 9PL, UK

2Climate and Global Dynamics Laboratory, NSF National Center for Atmospheric Research, Boulder, CO 80307, USA

* Correspondence to: Yuan Sun (yuan.sun@manchester.ac.uk), Zhonghua Zheng (zhonghua.zheng@manchester.ac.uk)

Contents of this file

Tables S1 to S2

Introduction

This supporting information contains details of single-point simulations, including urban parameter input (Table S1) and output of anthropogenic heat flux (Table S2).

| Parameter name | Long name | Unit | Site name |  |
| --- | --- | --- | --- | --- |
|  |  |  | FR-Capitole | UK-Manchester |
| CANYON_HWR | Canyon height-to-width ratio | Unitless | 1.32 | 0.75 |
| HT_ROOF | Height of roof | meter | 15 | 26 |
| NLEV_IMPROAD | Number of impervious road layers | Unitless | 2 |  |
| THICK_ROOF | Thickness of roof | meter | 0.14 | 0.15 |
| THICK_WALL | Thickness of wall | meter | 0.29 |  |
| WIND_HGT_CANYON | Height of wind in canyon | meter | 7.5 | 13 |
| WTLUNIT_ROOF | Fraction of roof | Unitless | 0.62 | 0.35 |
| WTROAD_PERV | Fraction of pervious road out of total canyon floor | Unitless | 0.26 | 0.69 |
| ALB_IMPROAD_DIF/ ALB_IMPROAD_DIR | Diffuse/direct albedo of impervious road | Unitless | 0.13 |  |
| ALB_PERROAD_DIF/ ALB_PERROAD_DIR | Diffuse/direct albedo of pervious road | Unitless | 0.13 | 0.08 |
| ALB_ROOF_DIF/ ALB_ROOF_DIR | Diffuse/direct albedo of roof | Unitless | 0.18 | 0.23 |
| ALB_WALL_DIF/ ALB_WALL_DIR | Diffuse/direct albedo of wall | Unitless | 0.23 | 0.27 |
| EM_IMPROAD | Emissivity of impervious road | Unitless | 0.97 | 0.91 |
| EM_PERROAD | Emissivity of pervious road | Unitless | 0.99 | 0.94 |
| EM_ROOF | Emissivity of roof | Unitless | 0.92 | 0.89 |
| CV_IMPROAD | Volumetric heat capacity of impervious road | kJ/m3 K | [2060.5, 1712.3] |  |
| CV_ROOF | Volumetric heat capacity of roof | kJ/m3 K | [1957.2, 994, 994, 1.2, 1.2, 1.2, 10.08, 10.08, 10.08, 609] | [1700, 1.2, 994, 1.2, 1.2, 1.2, 10.08, 10.08, 10.08, 609] |
| CV_WALL | Volumetric heat capacity of wall | kJ/m3 K | [1524, 1525, 166, 918, 772, 771, 772, 227, 204, 628] | [1521, 1521, 138, 919, 773, 773, 773, 226, 194, 621] |
| TK_IMPROAD | Thermal conductivity of impervious road | W/m K | [1.67, 0.56] |  |
| TK_ROOF | Thermal conductivity of roof | W/m K | [1.15, 0.15, 0.15, 0.03, 0.03, 0.03, 0.04, 0.04, 0.04, 0.16] | [1.2, 0.03, 0.15, 0.03, 0.03, 0.03, 0.04, 0.04, 0.04, 0.16] |
| TK_WALL | Thermal conductivity of wall | W/m K | [2.03, 6.15, 5.85, 6.21, 4.77, 0.66, 4.77, 5.7, 5.85, 1.81] | [2.52, 2.52, 0.15, 2.11, 0.68, 0.68, 0.68, 1.6, 2.23, 2.3] |
| T_BUILDING_MIN | Minimum indoor temperature | °C | 11.95 | 16.95 |
| T_BUILDING_MAX | Maximum indoor temperature | °C | 26.85 | 26.85 |
| p_ac | Air conditioning penetration rate | Unitless | 0.047 | 0.018 |

1 At FR-Capitole, urban parameters are from Urban-PLUMBER's detailed experiment. Among them, emissivity parameters are derived from CLM5.0's default dataset. That is, EM_IMPROAD, EM_PERROAD, and EM_ROOF were 0.97, 0.99, and 0.92, respectively. In the new dataset used for the CTSM development version, these values have been updated to 0.91, 0.95, and 0.91.

Table S1. Urban parameters.

| AHF (unit: W/m2) | Site Name |  |
| --- | --- | --- |
|  | FR-Capitole | UK-Manchester |
| CNTL simulation | 6.25 in 2004 | 9.99 in 2022 |
| TRAF simulation | 27.72 in 2004 | 25.86 in 2022 |
| AH4GUC for the 2010s (Varquez et al., 2021) | 41.78 | 21.4 |
| Jin et al. (2019)’s global gridded dataset for 2015 | 19.6 | 29.9 |
| AH-DMSP for 2010 (Yang et al., 2017) | 0.1 | 0.6 |

Table S2. List of annual mean anthropogenic heat flux (AHF).